Subject: Television and Video Engineering

2 Mark Questions and Answers

1. Mention  the major function  of the camera tube?

The  major  function  of  the  camera  tube  is  to  convert  an  optical  image  into electrical  signals.

2. Define visual  acuity?

Visual  acuity  can  be  defined  as the  ability  of  human   eye  to  resolve  finer details  in  a  picture

3. What  do  you  refer by persistence of eye?

The persistence of eye  refers  to the storage  capability of the human  eye

4.Define luminous  intensity?

It   is the   measure   of   light   intensity   responsible   for   stimulating   visual sensation.

5.Define aspect  ratio?

Aspect  ratio  can be  defined  as  the   ratio  of  width  to  height  of  the picture frame.  For television, it  is  standardized  as  4:3.

6.Define luminous  flux?

Luminous  flux  can  be  defined  as  the  radiated  luminous  power or  power  of visible light expressed  interms  of its  effect  on  the average or normal  human  eye.

7.Define luminance?

Luminance  can  be  defined  as  the  quantity  of  light  intensity  emitted  per square  centimeter of  an illuminated  area.

8.What  do you  understand  by illuminance?

Illuminance is  the average luminous  flux  incident  on  to  a surface.

9.what  do  you  mean  by resolving power or  resolution?

It  is  the  ability  of  the  image  reproducing  system  to  represent  the  fine structure of  an  object.

10.Mention  some important  characteristics  of human  eye?

Visual  acuity,  persistence  of  vision,  brightness  and  colour  sensation  are some of the important  characteristics  of human  eye.

11.What  are rods and  cones?

The  retina  of  the  human  eye  consists  of  light  sensitive  cellular  structures of  two  kinds  namely  rods  and  cones.  The  rods  sense  primarily  the  brightness levels   including   very   faint   impressions.   The   cones   are   mainly   responsible   for colour  perception.  There  are  65  lakhs  cones  and  about  10  crores  rods  connected to  the brain through  8  lakhs  optic nerve fibres.

12.Give the relationship  between eye parameters?

The eye parameters  are  related  by the  following equation:

B=constant / c2α02

Where,

c- the constant  ratio

b-the scene brightness

α0   - the minimum  resolvable angle

13.Why is  scanning necessary in  television  system?

Scanning  is  the  important  process  carried  out  in  a  television  system  inorder to obtain  continuous  frames  and  provides   motion  of picture.  The  scene  is  scanned both  in  the  horizontal  and  vertical  directions  simultaneously  in  a  rabid  rate.  As a result  sufficient  number  of  complete  picture  of  frames  per  second  is obtained  to give the illusion  of continuous  motion.

14.What do  you  understand  by flicker?

The result  of  24  pictures per  second  in  motion  pictures and  that  of  scanning

25 frames  per  second  in  television  pictures  is  enough  to  make  an  illusion  of continuity.  But,  they  are  not  rapid  enough  to  permit  the  brightness  of  one  picture or  frame  to  blend  smoothly  in  the  next  through  the  time  when  the  screen  is blanked  between  successive  frames.  This  develops  in  a definite  flicker of  light  that is  very  irritating   to  the  observer  when  the  screen  is  made  alternately  bright  and dark.

15.How  will  you  solve  the flickering problem?

The   flickering   problem is   solved   in   motion   pictures   by   showing   each picture  twice.  Hence  48  views  of  the  scene  are  shown  per  second  although  they are  still  the  same  24 pictures  frames  per second.  As  a result  of  the  increased blanking rate,  flicker is  removed. In TV, Interlaced scaning is used to avoid flicker

16.What  do  you  mean  by interlaced  scanning?

When   the   scanning   beam   reaches   the   bottom   of   the   picture   frame,   it quickly  returns  to  the  top  to scan  those  lines  that  were  missed   in  the  previous

scanning.  Hence  the  total  number  of  lines  are  divided  into  two  groups  called fields. Each field is scanned alternately. This way of scanning is called  interlaced scanning.

17.Define vertical  resolution?

The  capability  of  resolving  picture  details  in  the  vertical  direction  is  called vertical  resolution.

18.What  is  horizontal  resolution?

The  ability  of  the  system  to  resolve  maximum  number  of  picture  elements along the scanning  lines  determines  horizontal   resolution.

19.List  the contents  of a composite video  signal?

Composite video signal consists of a camera signal, blanking pulses and synchronizing pulses.

20.What  do  you  mean  by pedestal?

The  difference  between  the  black  level  and  blanking  level  is  known  as  the pedestal.

21.Define peak-white level?

The  peak-white  level  is  defined  as  the  level  of  the  video  signal  when   the picture   detail   being   transmitted    corresponds   to   the   maximum   whiteness   to be handled

22.Define pedestal  height?

Pedestal  height  is  the  distance  between  the  pedestal  level  and  the  average value axis  of  the video   signal.

23.What  is  the main  function  of the blanking  pulses?

The  composite  video  signal  consist  of  blanking   pulses  to  make  the  retrace lines  invisible  by  increasing  the  signal  amplitude  little  above  the  black  level  of

75percent  during the time  scanning the  circuits  develop  retrace.

24.What  are per the post  equalizing pulses?

To  rectify  the  drawback  which  occurs  on  account  of  half-line  discrepancy five  narrow  pulses  are added  on   either  side  of  the  vertical  sync  pulse. These are called  pre-equalising  and  post-equalising pulses .

25. Differentiate monochrome and colour camera tube.

In black and white camera only one pickup tube is needed but three such tubes or its equivalent is necessary  in color cameras to develop separate signals for red ,green and blue information present in the scene.

26.     Why do we prefer horizontal polarization for television receiving antenna?

We prefer horizontal polarization for television receiver antenna because it results in more signal  strength,  less  reflection  and  reduced  ghost  images.

27.Where can  you  employ indoor receiver antennas?

In strong  signal  areas  it  is  sometimes feasible  to  use  indoor  antennas provided  the receiver is  sufficiently sensitive.

28.A  yagi  antenna  with  a  large number of directors  is  commonly used  with success  in  fringe areas  for stations  in  the vhf band.

29.What  do  you  understand  by diplexer?

The   outputs   of   both   the   video   and   the   audio   transmitter   are combined   by   the   diplexer   circuit   and   given   to   a   common   broadcast transmitting antenna.

30.   If two  stations  are  operating at  the same  carrier frequency and  located nearby then  they will  interface with  each  other.  This  is  called  co-cannel interference  and  it  is  common   in  fringe   areas.

31   It  is  the interference due to  stations  located  nearby and  allocated  as adjacent   channels.

32.Define  guard  band?

Guard  band   can  be   defined  as   a  small  frequency  band  introduced between  two  consecutive  channels  inorder to  reduce interference .

33. Ghost  interference  arises  as  a result  of discrete reflections  of the signal from  the  surface of hills,  bridges,  buildings,  towers  etc.

34.Mention  the requirements  of high  level  modulation?

In  high  level  modulation,  the  video  signal  has  to  be  modulated  by the   picture  carrier  in  the  final  power  amplifier  which  has  a high  power level.  Grid  bias  modulation  is  employed.

35.What  do  you  understand  by ground  waves?

Vertically  polarized  electromagnetic  waves  are  radiated  at  zero  or small  angles  with  ground.  They  are  guided  by  the  conducting  surface  of the  ground  along  which  they  are  propagated.  Such  waves  are  known  as ground  or  surface  waves.  As  the  ground  waves  travel  along  the  surface  of the  earth,  their  attenuation  is  proportional  to  frequency.  The  attenuation is reasonably low below 1500KHz.Therefore, all medium wave broadcast and long wave telegraph  and  telephone communication  is  carried  out  by ground  wave propagation.

36.What are sky waves?

In ground wave propagation ,frequencies above 1600 KHz does not serve any useful purpose as the signal gets very much attenuated within a short distance of its transmission .Therefore ,most radio communication in short wave bands upto 30MHz is carried out by sky waves.When these waves are transmitted high up in the sky , they travel in the straight line until the ionosphere is reached. This region begins about 120Km above the surface of the earth. The region consists of large concentrations of charge gaseous ions, free electrons and neutral molecules. The ions and free electrons cause to band all passing electromagnetic waves.

37.Describe briefly about space wave propagation.

Propagation of radio waves above about 40MHz is not possible through either sky wave or surface wave propagation .Therefore ,the only alternative for transmission in the VHF and UHF bands, despite large attenuation   is by radio waves which travel in a straight line from transmitter to receiver. This called space wave propagation.

38.Why is AM preferred over FM broadcasting the picture signal?

If FM is adopted for picture transmission ,the changing beat frequency between the multiple paths delayed with respect to each other would develop  a bar interference in the image with a shimmering effect as the bars continuously changes as the beat frequency changes therefore ,no study picture is produced.Apart from that ,circuit complexity and BW requirements are much less in AM than FM.Hence AM is preferred to FM for broadcasting the picture signal.

39.What is Dipole array?

Dipole antenna is used for band I&III transmitters. It consists of diploe pnels mounted on the four sides at the top of the  antenna tower.Each panel has an array of full wave dipoles mounted in front of reflectors. To get an unidirectional pattern ,the four panels mounted on the four sides of the tower are so fed that the current in each lags behind the previous by 90 degree . This is done by changing the field cable length by λ/4 to the two alternate panels and by reversal of polarity of the current.

40. Define Image rejection ratio.

Image rejection ratio is defined as the output due to desired station divided by output due to image signal.

41. What do you refer by Yagi uda Antenna?

This  is  a  widely used  antenna  for  television  receivers  .Generally ,for locations within 40 to 60Km from the transmitter is the folded dipole with one reflector and one director. This is commonly called Yagi antenna or Yagi-Uda antenna.

42. Name the essential parts of TV transmitter.

The essential parts of TV tranmitter includes a video processing unit . A visual modulator which is a diode bridge modulator, phase compensator or delay equalizer and frequency converter.

43. What is the main purpose of using VHF tuner?

The purpose tuner unit is to amplify both picture and sound signals picked up the antenna and to convert the carrier frequencies and their associated side bands into intermediate frequencies.

44. Name the essential components of RF section.

RF tuner section consists of RF amplifier ,mixer and local oscillator and is normally mounted on a separate subchasis,called the front end.

45. What are the major tasks to be done by detector?

The video detector is designed to recover composite video signal and to transform the sound signal to another lower carrier frequency.

46. Why is video amplifier required?

The  amplitude  of  the  composite  video  signal  at  the  output  of  video detector is not enough to drive the picture tube directly . Therefore further amplification is required. This is done by video amplifier.

47. What are Sync pulses?

The Synchronizing pulses called ‘Sync’ are part of the composite video signal as the top 25% of the signal amplitude. The sync pulses include horizontal

,vertical and equalizing pulses. Sync separator separates these signals from the video signal.

48. Define frequency Distortion.

The inequality in gain at different frequency components of the received signal is called frequency distortion.

49. List out the advantages of IF sections.

The main function of this section is to amplify modulated IF signal over its entire bandwidth with an input of about 0.5mV signal from the mixer to deliver about 4V into the video detector.

IF section is used to equalize amplitudes of sideband components ,because   of vestigial side band transmission.

IF section is used to reject the signals from adjacent channels.

50. What is meant by high level modulation.

In high level modulation modulation occurs in the output circuit of the final amplifier.

51. What do you understand by Hue?

Hue or tint can be defined as the predominant spectral colour of the received light. The colour of any object is distinguished by its hue or tint.

52 Define Brightness.

Brightness can be defined as theamount of light intensity as perceived by the eye regardless of the colour.

53. What do you mean by saturation?

Saturation refers to the spectral purity of the colour light.It indicates the degree by which the colour is diluted by white.

54. List any three requirements to be satisfied for compatibility in television systems.

a.   It  should  has  the  same  bandwidth  as  the  corresponding  monochrome

signal.

b.   The colour signal should have the same brightness information as that of monochrome signal.

c.   The location and spacing of the picture and sound carrier frequencies should remain the same.

55. What is additive mixing?

All light sensations to the eye are splitted in to three main colour groups namely red,blue and green. The optic nerve system integratesthe different colour impressions in accordance with the curve to perceive the actual colour of the object.

56. State grassman’s law.

The brightness impression produced by the three primaries that constitute the single light. This property of the eye of generating a response which depends on the algebraic sum of the blue ,red and green inputs is called grassman’s law.

57. Explain the significance of generating colour difference signals.

Colour difference signals are generated to avoid the separate transmission of R,G,B signals.

58. Why is (G-y) not suitable for transmission?

The proportion of G is large in luminance signal,hence magnitude of (G- Y) is relatively small so it requires amplifiers at the receiving end.

It affect the signal to noise ratio at the transmitting end.

59. What is gamma correction.

A colour camera is used develop three voltages proportional to red,green and blue colour  contents  of  the  picture.These  voltages  are  represented  as  R,G,B.a correction is applied to these voltages to compensate for any nonlinearity  of the system and that of the picture tube.This is called gamma correction.i.e. the camera tube output voltage amplitudes are normalized to I V p-p level.

60. what do you mean by compatibility?

Compatibility means that a colour TV signal can produce a black and white picture on a monochrome receiver and signals from a black and white system can provide a monochrome picture on a colour receiver.

61. What do you mean by colour burst?

In PAL system the two carrier components are suppressed in the balanced quadrature   modulator   it   is   necessary   to   regenerate   at   the   receiver   for demodulation .For this ,8 to 10 cycles of the colour subcarrier oscillator output at the encoder are transmitted along with other sync pulses. This sample of the colour subcarrier called colour burst,is placed at the back porch of each horizontal blanking pulse pedestal.

62. What is swinging burst?

The PAL burst phase actually swings 45 about the –U axis from line to line and indicates  the  same  sign  as  that  of  the  V  signal;thus  the  switching  mode information is the swinging burst.this is known as swinging burst.

63. Merits of SECAM system.

SECAM  system has several advantages because of frequency modulation of the subcarrier and transmission of one line at a time.

SECAM receivers are immune to phase distortion.

Both the luminance and chrominance signals are not present at the same time ,there is no possibility of cross talk between the colour difference signals.

Ther is no need for the use of QAM at the transmitter and synchronous

detectors at the receiver.

The receiver does not need Automatic tuning control and Automatic color control

64. Demerits of SECAM system.

In SECAM system luminance is represented by the amplitude of voltage but hue and  saturation  are represented  by deviation of the subcarrier.when  a composite signal involving luminance and chrominance is faded out in studio operation,it is the luminance signal that is readily attenuated and not6 the chrominance.This makes the color more saturated during fade to black.

65. Limitations of the NTSC system.

The NTSC  system  is  sensitive to  transmission  path  differences  which introduces phase errors that result in colour changes in the picture. At the transmitter,phase changes  in  the chroma signal  take place when  change over between programmes of local and television network systems takes place and when video tape recorders are switched on .The phase angle is also affected by the level of the signal while passing through various circuits .In addition cross talk between demodulator outputs at the receiver causes colour distortion.

66. Mention some features of PAL system.

a.   The weighted (B-Y) and (R-Y)  signals are modulated without being given a phase shift of 33 as is done in the NTSC system.

b.   On modulation both the color difference signals are allowed the same bandwidth of about 1.3MHz.

c.   The color subcarrier frequency is chosen to be 4.43MHz.

d.   The weighted color difference signals are quadrature modulated with the subcarrier.

67. Write notes on NTSC system.

NTSC system is compatible with 525 line American system.In order to maintain compatibility two new colour difference signals are generated and they are represented as I and Q. Since eye is capable of resolving finer details in the regions around I,it is allowed to have a maximum bandwidth of 1.5MHz.The bandwidth of Q signal is restricted to 0.5MHz.

68. What is the difference between NTSC,PAL and SECAM?

The difference between the SECAM system on one hand and NTSC and PAL on the other is that the later transmit and receive two chrominance signals simultaneously while in the SECAM system only one of the two color difference signal is transmitted at a time.

69. What do you mean by high frequency preemphasis?

In SECAM system, the chrominance signals are pre-emphasized before modulation. After modulating the carrier with the pre-emphasized and weighted color difference signals, another form of preemphasis is carried out on the signals. This takes the form of increasing amplitude of the sub carrier as its deviation increases. Such a preemphasis is called high frequency preemphasis.

70. What is the use of line identification pulses?

In SECAM system ,the switching of Dr and Db signals line by line takes place during  the line  sync  pulse period.The  sequence of  switching continues without interruption from one field to the next and is maintained through the field blanking interval.However it is necessary for the receiver to be able to deduce as to which line is being transmitted.Such an identification of the proper sequence of color lines in each field is accomplished by identification pulses.

71. Write notes on luminance channel.

The video amplifier in the luminance channel is Dc coupled and has the same bandwidth  as  in  the monochrome receiver.  It  is  followed  by a delay line to compensate for the additional  delay the color signal suffers because of  limited bandpass of the chrominance amplifier .This ensures time coincidence of the luminance and chrominance   signals. The channel also includes a notch filter which attenuates   the subcarrier by about 10db.This helps to suppress the appearance of any dot structure on the screen along with the color picture.

72. What is the use of chrominance bandpass amplifier?

The chroma bandpass amplifier selects the chrominance signal and rejects other unwanted components of the composite signal.

73. What do you mean by automatic color control?

The ACC circuit is similar to the AGC circuit used for automatic gain control of RF and IF stages of the receiver.It develops a dc control voltage that is proportional to the amplitude of the color burst.

74. Write short notes on color killer circuit.

When a monochrome transmission is received there is no input to the color killer and no positive voltage is developed . Therefore no input is given to the second chroma amplifier from the color killer circuit ,it blocks the second chroma amplifier.Thus it prevents the color noise on black and white picture.

75. Merits of PAL system.

The problem of differential phase errors has been successfully overcome in the PAL system.

76. Demerits of PAL system.

The use of phase alteration by line technique   and associated control circuitry together with the need of a delay line in the receiver makes the PAL system more complicated and expensive.The receiver cost is higher for the PAL colour system.

77. What do you mean by automatic frequency tuning?

AFT is used to improve the stability of the oscillator circuit ,some drift does occur on  account  of ambient  temperature changes  ,component  aging ,power  supply voltage fluctuation and so on. The fine tuning control is adjusted to get a sharp picture.

78. Write short notes on burst seperator.

The burst seperator circuit has the function of extracting 8 to 10 cycles of reference color burst which are transmitted on the back porch of every horizontal pulse .the circuit is tuned to the subcarrier frequency and is keyed on during the flyback time by pulses derived from the horizontal output stage.

79. What is the use of color subcarrier oscillator?

The  function  of  subcarrier  oscillator  is  to  generate  a  carrier  wave  output  at

3.57MHz and feed it to the demodulators.The subcarrier frequency is maintained at  its correct value and phase by the APC circuit.

80. How the phase error is cancelled in the PAL system.

In PAL system phase shift error is cancelled by reversing the phase angle of v signal  on alternate lines.

81. Give the abbreviation for NTSC, SECAM ,and PAL.

NTSC  -National Television systems committee

SECAM –Sequential –a-Memoire

PAL - Phase Alteration by Line

82. What do you understand by PAL –D Colour system.

The use of eye as the averaging mechanism for the correct hue is the basic concept of simple ‘PAL’ system. Beyond a certain limit , the human eye see the effect of colour changes on alternate lines hence the system needs modification. Considerable improvement found in the system  of a delay line is used to do the averaging first and then present the color to the eye.This is called PAL-D or delay line PAL method and is most commonly employe in PAL receivers.

83. Write short notes on colour subcarrier frequency of PAL D system.

The color sub carrier frequency of 4.43MHz is produced with a crystal controlled oscillator  .To  accomplish  minimum  raster     disturbance  through  the  color subcarrier it is important to maintain correct frequency relationship between the scanning frequencies and subcarrier frequency .Therefore ,it is usual to count down from the subcarrier frequency to twice the line frequency pulses .

84. write short notes on AGC circuit.

AGC circuit is used to control the gain of RF and IF amplifiers .The change in gain is achieved by shifting the operating point of transistors used in the amplifiers.The operating point is changed by a bias voltage that is developed in the AGC circuit.

85. What do you mean by Peak AGC system?

The system based on sampling the sync tip levels is known as “peak” AGC system.The Peak AGC system is also called as non-keyed AGC sytem.

86. What are the two types of AGC control?

Forward AGC control

Reverse AGC control

87. What is Forward AGC control?

In  any  transistor  amplifier  ,gain  is  varied  by  shifting  the  operating  point  either towards  collector  current  cutoff  or  saturation.  This  actually  varies  beta  of  the transistor and hence the stage gain changes.When gain is changed by shifting the operating point towards current cutoff ,then it is called “Reverse AGC”.

88. What is forward AGC?

In  any  transistor  amplifier  ,gain  is  varied  by  shifting  the  operating  point  either towards  collector  current  cutoff  or  saturation.  This  actually  varies  beta  of  the

transistor and hence the stage gain changes.When gain is changed by shifting the operating  point  towards  collector  current  saturation    ,then  it  is  called  “Forward AGC”.

89. List the draw backs of nonkeyed AGC.

The AGC voltage developed across the peak rectifier load tends to increase during vertical sync pulse periods because the video signal amplitude remains almost at the peak value every time vertical sync pulses occur. This results in a 50Hz ripple over the  negative AGC voltage and reduces gain of the receiver during these intervals.The reduced gain results in weak vertical sync pulse which in turn can put the vertical deflection oscillator out of synchronism causing rolling of the picture.

90. Merits of Keyed AGC system.

AGC voltage developed is a true representation of the peak of fixedsync level and thus corresponds to the actual incoming signal strength.

Noise effects are minimized because conduction is restricted to a small fraction of the total line period.

91. Write short notes on CATV.

CATV stands for community antenna television systems. The CATV system is a cable system distributes good quality television signal to a very large number of receivers throughout an entire community. Generally this system gives increased TV programmes to subscribers who pay a fee for this service. A cable system may have many more active VHF and UHF channels than a receiver tuner can directly select.

.

92. What do you understand  by satellite TV?

Satellite TV is a TV from space. Broadcasters from earth transmit their programmes to specified satellites. Then, the transmissions are returned to receiving equipment on the ground. Therefore, the better the receiving equipment

,the higher the quality of the reception.

93. List some of the  applications of satellite TV?

The shear range of programmes currently available on satellite channel is very much impressive such as 24-hour music videos, news, and feature films. A variety of general entertainment programmes, sports ,children’s programmes , foreign language broadcasts and cultural programmes are all available for the keyboard dish owner. Some of these come through subscription channels and others by free to watch channels which are sponsored by advertising.

94. State the merits of satellite TV.

The picture quality from satellite systems is surprisingly good and compares well with conventional land based TV transmissions. In addition unlike terrestrial broadcasts it is free from the spectrum of picture ghosting.

95. Mention the limitations of satellite TV.

The limitation of the satellite TV is varying picture and audio quality .By adopting de-emphasis circuits we can avoid the variations.

96. Give the applications of video tape recorders.

Smaller and  lower priced  video  tape  recorders  using ½  inch  tape  are available for closed type circuit TV or for use in the home. They can record and playback programs on a television receiver in color and monochrome. In addition to that small portable cameras are provided for a complete television system with the recorder. These portable systems are also employed for taping   television programs from a remote are also employed for taping television programs from a remote location for away from the TV broadcast studio.

97. List some merits of high definition television.

Improvement in both vertical and horizontal resolution of the reproduced picyure by approximately 2:1 over existing standards.

Much improved colour rendition

Higher aspect ratio of atleast 5:3

Stereophonic sound

98. What do you mean by Longitudinal video recording?

A method in which video signals are  recorded on atleast  several tracks along the length of the tape.

99. What do you mean by Quadruplex (Transverse) scan recording?

In transverse scan recording ,four recording heads are spaced 90 apart and are mounted on a rotating drum and the tape moves past it,transversely.Each head comes in contact with the tape as the previous one leaves it.

100. What do you mean by helical scan recording ?

In helical scan recording ,the two recording heads ‘look at ‘ the tape surface as it is drawn past them  through two tiny rectangular slits mounted on opposite sides of the drum.The heads thus trace out diagonal tracks across the tape,one track per head.

| 101. |  | What are two types of video disc system? Laser or optical disc system |
| --- | --- | --- |
|  |  | Capacitance disc system |
| 102. |  | List the fundamental components of DVD player. A drive motor to spin the disc. |
|  |  | A laser and lens system to focus in on the bumps and read them. |

|  |  | Tracking mechanism that can move the laser assembly so that the lasers |
| --- | --- | --- |
|  |  | beam can follow the spiral track. Electronic circuitry |
| 103. |  | What are the advantages of DVD players over VCR’s. |
|  |  | The quality of picture and sound in a DVD is better than on a video |

tape,and  DVD’s    maintain  their  high  quality  over  time,because  there  is  no

physical contact with the disc as it revolves.

104.           List 4 merits of digital TV receivers.

o Reduced Ghosts

o Reduction of 50Hz flicker

o High resolution pictures

o Slow motion action

PART B

1.   Describe how the flicker is solved  by interlaced scanning?

Interlaced Scanning

Effective rate of 50 vertical scans per second is utilized to reduce the flicker.

Increase the downward rate of travel of the scanning electron beam.

Every alternate line gets scanned  instead of every successive line. Scanning Periods

Duration of one horizontal line is 64µs

Duration of one vertical trace is 20ms

2.   Explain the structure and the generation of video output from a vidicon camera.

•   it functions on the principle of photo conductivity

•	in photoconductive method where the conductivity or resistivity of the photoconductive surface varies with in proportion to the intensity of light focused on it.

•	With a B+ source of 40V ,an area with high illumination may attain a potential of about +39V on the beam side.

•   Dark areas may attain a potential of about +35 on the beam side.

•	As  the  beam  scans  the  target  plate,  it  encounters  different  positive potentials on the side of photo layer that faces the gun.

•	Number of electrons from the beam is then deposited on the photo layer surface to reduce to reduce the potential of each element.

•	Sudden change in potential on each element causes a current flow in the signal electrode circuit producing a varying voltage across the load resistance.

3.   Give the constructional details of a monochrome picture tube and explain the beam landing

Employs electrostatic focusing and electro magnetic deflection

Composite video signal that is injected either at the grid or cathode of the tube

Electron Gun:-emits the electron beam with proper energy to cause it to fluoresce, another anode called final anode is included within the tube.

Low Voltage Focusing:

It needs less voltage at the focusing electrodes. High Voltage Focusing:

It requires high voltage at the focusing electrode. This focus system is called ‘unipotential lens’ system.

Picture Tube Screen:

Screen phosphor: -light metals such as zinc and cadmium in the form of sulphide, sulphate, and phosphor compounds are used.

Screen Brightness:-picture tubes employ a very thin coating  of aluminium on the back surface of the screen .This improves the screen brightness.

4.   Explain in detail the silicon diode array vidicon camera tube.

It uses photovoltaic type where the target is prepared from a thin n-type silicon wafer instead of deposited layers on the glass faceplate.

Oxidation:-to form a film of silicon dioxide(Sio2)which is an insulator.

Photomasking and etching: - an array of fine openings is made in the

Oxide layer.

Boron is vapourized   through the array of holes.

Thin layer of gold is deposited on each p type opening to form contacts for signal output.

The silicon target plate thus formed is typically 0.003cm thick,1.5 cm square having an array of 540x540 photodiodes.

The Target plate is mounted in a vidicon type of camera tube.

5.   Write notes on composite video signal.

Composite  video  signal  consists  of  a  camera  signal,  blanking  pulses and  synchronizing pulses.

The  difference  between  the  black  level  and  blanking  level  is  known as  the pedestal.

The  peak-white  level  is  defined  as  the  level  of  the  video  signal when     the   picture   detail   being   transmitted     corresponds   to   the maximum  whiteness  to be handled

Pedestal  height  is  the  distance  between  the  pedestal  level  and  the average value  axis  of  the video   signal.

The  composite  video  signal  consist  of  blanking   pulses  to  make  the retrace  lines  invisible  by  increasing  the  signal  amplitude  little  above the  black  level  of  75percent  during  the  time   scanning  the   circuits develop  retrace.

To   rectify   the   drawback   which   occurs   on   account   of   half-line discrepancy   five   narrow   pulses   are added   on    either   side   of   the vertical    sync    pulse.  These  are    called    pre-equalising      and    post- equalising pulses .

6.   Draw  Block  Diagram  of  RF  Tuner  and  explain  how  incoming  signals  from different sections are translated to common picture IF and sound IF frequencies.

RF tuner section consists of RF amplifier ,mixer and local oscillator and is normally mounted on a separate subchasis,called the front end.

The purpose tuner unit is to amplify both picture and sound signals picked up the antenna and to convert the carrier frequencies and their associated side bands into intermediate frequencies.

The   receiver   uses   superheterodyne   principle.The   signal   voltage   or information from various stations modulated over different carrier frequencies  is  heterodyned  in  the  mixer  with the  output  from  a  local oscillator to transfer original information on a common fixed carrier frequency called intermediate frequency(IF)

The  setting  of  local  oscillator  frequency  enables  selection  of  desired station.

7.   Describe briefly the factors that influenced the choice of picture IF =38.9 and sound IF =33.4MHz in the 625 line system.

The factors that influence the choice of intermediate frequencies in TV receivers are:

Image Rejection Ratio:

Image rejection ratio is defined as the output due to desired station divided by output due to image signal.

Pickup due to local oscillator radiation from TV receivers

Image Frequencies should not lie in the FM band

Interference or direct pickup from bands assigned for other services

Gain

8.   Explain how composite video signal is detected ?How is the polarity of video output signal decided?

Modulated IF signals after  due amplification in the IF section are fed to the video detector .

The video detector is designed to recover composite video signal and to transform the sound signal to another lower carrier frequency.

This is done by rectifying the input signal and filtering out unwanted frequency components.

A diode is used ,which is suitably polarized to rectify  either positive or negative peaks of the input signal.

9.   Draw block  diagram of   a monochrome TV  receiver and briefly explain the operation of TV receiver.

A black and white  receiver has six main sections:-

Tuner section, IF sub system ,Video section ,Audio section, Deflection and power supply.

Tuner section:

This section consists of RF amplifier ,mixer and local oscillator.It selects the desired channel signal from those picked up by the antenna and converts it into the IF band.

IF subsystem:

The IF subsystem   includes IF section ,AGC circuit and video detector.The IF section provides large amplification to the weak IF signals and the video detector that follows IF amplifiers produces composite  video and intercarrier FM sound signals.

Video section:

The Video signal is accorded large amplification and fed to the picture tube. Audio section:

The sound associated with the picture is transmitted as an FM signal with a center frequency 5.5MHz above the station picture carrier frequency. The sound signal BW is only about 150Khz as compared to the large bandwidth of 5MHz for the picture signal.

Sync separation and processing:

The horizontal   and vertical sync pulses that form part of the composite video signal are separated in the sync separator. The composite video signal is either taken from the video detector output or after one stage of video amplification.

DC power supplies:

All the active devices ,ICs  and picture tube need a dc source for their operation. Rectifiers are used to convert ac to dc and filters provided to eliminate ac ripple in the dc output.

10. Explain briefly the operation of IF subsystem.

The   IF   subsystem      includes   IF   section   ,AGC   circuit   and   video detector.The IF section provides large amplification to the weak IF signals and the video  detector  that  follows  IF  amplifiers  produces  composite     video  and intercarrier FM sound signals.

The factors that influence the choice of intermediate frequencies in TV receivers are:

Image Rejection Ratio:

Image rejection ratio is defined as the output due to desired station divided by output due to image signal.

Pickup due to local oscillator radiation from TV receivers

Image Frequencies should not lie in the FM band

Interference or direct pickup from bands assigned for other services

Gain

IF section :

The main function of this section is to amplify modulated IF signal over its bandwidth   with an input of about 0.5mV signal from the mixer to deliver about 4V into the video detector.

IF section is used to equalize amplitudes of sideband components ,because of vestigial side band transmission.

IF section is used to reject the signals from adjacent channels.

11.       Explain in detail the NTSC color receiver system.

NTSC system is compatible with 525 line American system.In order to maintain compatibility two new colour difference signals are generated and they are represented as I and Q. Since eye is capable of resolving finer details in the regions around I,it is allowed to have a maximum bandwidth of 1.5MHz.The bandwidth of Q signal is restricted to 0.5MHz.

Limitations of the NTSC system.

The NTSC system is sensitive to transmission path differences which introduces phase errors that result in colour changes in the picture. At the transmitter,phase changes in the chroma signal take place when change over between programmes

of local and television network systems takes place and when video tape recorders are switched on .The phase angle is also affected by the level of the signal while passing through various circuits .In addition cross talk between demodulator outputs at the receiver causes colour distortion.

12. Explain working of a pAL system.

The weighted (B-Y) and (R-Y) signals are modulated without being given a phase shift of 33 as is done in the NTSC system.

On modulation both the color difference signals are allowed the same bandwidth of about 1.3MHz.

The color subcarrier frequency is chosen to be 4.43MHz.

The weighted color difference signals are quadrature modulated with the subcarrier.

13.Describe with necessary diagrams the encoding of color difference signal.

Quadrature modulation:

It is used to combine two colour difference signals in to a single variable. Frequency interleaving:

The technique of combining two distinct signal within the same bandwidth is known as frequency interleaving.

Colour burst:

Sample of colour subcarrier output at the encoder are transmitted along with other sync pulses.

14.Describe the generation of Y signal and color difference signals.

Find the value for cyan.

In a color camera ,the luminance or Y signal is obtained by adding R,G

and B in the following proportions

Y=0.30R+0.59G+0.11B.

15.Explain in detail about SECAM system.

SECAM color system is compatible wit819 line monochrome system.

The difference between the SECAM system on one hand and NTSC and PAL on the other is that the later transmit and receive two chrominance signals simultaneously while in the SECAM system only one of the two color difference signal is transmitted at a time.

In SECAM system, the chrominance signals are pre-emphasized before modulation. After modulating the carrier with the pre-emphasized and weighted color difference signals, another form of preemphasis is carried out on the signals. This takes the form of increasing amplitude of the sub carrier as its deviation  increases. Such a preemphasis is called high frequency preemphasis.

16.Explain the working of U and V demodulators.

The chrominance available at the output of delay line circuitry consists of two suppressed carrier  amplitude modulated components designated as U and V.

These correspond to the two color difference signal and bear information about their amplitude and polarity

Because of quadrature modulation ,the two modulation product signals have a phase difference of 90 at any instant with respect to each other.

Thus when one is passing through its positive or negative peak, the other is passing through Zero.

The demodulating devices are normally biased to cutoff and conduct only at positive peaks of the externally fed subcarrier which is large in amplitude.

17.Describe with a circuit Burst phase discriminator.

The  burst  seperator  circuit  has  the  function  of  extracting  8  to  10  cycles  of reference color burst which are transmitted on the back porch of every horizontal pulse. The circuit is tuned to the subcarrier frequency and is keyed on during the flyback time by pulses derived from the horizontal output stage.

18.Write the need and working of Automatic Gain control Circuit.

AGC circuit controls the gain of RF and IF stage to enable almost constant signal voltage at the output of video detector, despite changes in the signal picked up by the antenna. The change in gain is achieved by shifting the operating point of transistors   used in the amplifiers. The operating point is changed by a bias voltage that is developed in the bias circuit.

19.Write Short notes on color killer circuit.

When a monochrome transmission is received there is no input to the color killer and no positive voltage is developed . Therefore no input is given to the

second chroma amplifier from the color killer circuit ,it blocks the second chroma amplifier.Thus it prevents the color noise on black and white picture.

20.Briefly explain PAL-D system.

The use of eye as the averaging mechanism for the correct hue is the basic concept of simple ‘PAL’ system. Beyond a certain limit , the human eye see the effect of colour changes on alternate lines hence the system needs modification. Considerable improvement found in the system  of a delay line is used to do the averaging first and then present the color to the eye.This is called PAL-D or delay line PAL method and is most commonly employe in PAL receivers.

21.Briefly explain satellite based TV broadcast system.

Satellite TV is a TV from space. Broadcasters from earth transmit their programmes to specified satellites. Then, the transmissions are returned to receiving equipment on the ground. Therefore, the better the receiving equipment

,the higher the quality of the reception. Applications of satellite TV:

The shear range of programmes currently available on satellite channel is very much impressive such as 24-hour music videos, news, and feature films. A variety of general entertainment programmes, sports ,children’s programmes , foreign language broadcasts and cultural programmes are all available for the keyboard dish owner. Some of these come through subscription channels and others by free to watch channels which are sponsored by advertising.

Limitation:

The  limitation  of  the  satellite  TV  is  varying  picture  and  audio  quality  .By adopting de-emphasis circuits we can avoid the variations.

22.Write short notes on Video disc system.

The video disc is essentially a gramophone record with pictures on it.The first video disc was the laser optical and the second video disc system is based on the capacitance principle.

23.Write short notes on High definition Television.

It  aims at

Improvement in both vertical and horizontal resolution of the reproduced picyure by approximately 2:1 over existing standards.

Much improved colour rendition

Higher aspect ratio of atleast 5:3

Stereophonic sound

24. Briefly explain CATV system.

25. Briefly explain Digital TV transmission and reception.