Here are answers to the questions from the exam paper you uploaded:

### Question 1

(a) What was the first-generation cellular system called?
The first-generation cellular system was called Advanced Mobile Phone System (AMPS).

(b) Define roaming as in mobile communication and give an example.
Roaming is the ability of a mobile device to access and use the network services provided by a different network operator or service area, typically while the user is outside their home network.
Example: Using your mobile phone in another country through an agreement between your home network and the foreign operator.

(c) Mention any three differences between CSMA/CA and CSMA/CD.

- Collision handling:

  - CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance): Avoids collisions by transmitting only when the channel is clear.

  - CSMA/CD (Carrier Sense Multiple Access with Collision Detection): Detects collisions after they occur and retransmits data.

- Used in:

  - CSMA/CA: Wireless networks like Wi-Fi.

  - CSMA/CD: Wired networks like Ethernet.

- Mechanism:

  - CSMA/CA: Uses acknowledgment packets to confirm data reception.

  - CSMA/CD: Monitors the network to detect collisions.

(d) Tabulate at least four main differences between GSM and CDMA.

| Feature | GSM | CDMA |
| --- | --- | --- |
| Access Technique | Time Division Multiple Access | Code Division Multiple Access |
| SIM Requirement | Requires a SIM card | Does not necessarily require a SIM |
| Bandwidth Usage | Allocates a specific bandwidth | Shares bandwidth across users |
| International Roaming | Supported widely | Limited support |

(e) With graphs explain FDMA, TDMA, and CDMA.

- FDMA (Frequency Division Multiple Access): Divides the frequency spectrum into separate channels. Each user is assigned a unique frequency band.

- TDMA (Time Division Multiple Access): Divides time into slots. Each user is assigned a specific time slot within the same frequency band.

- CDMA (Code Division Multiple Access): Uses unique codes to distinguish users, allowing them to share the same frequency and time.

### Question 2

(a) Abbreviate the following as in GSM network structure:

- NSS: Network Switching Subsystem

- ELR: Equipment Location Register

- BSC: Base Station Controller

- GMSC: Gateway Mobile Switching Center

(b) Differentiate between Um interface and Abis interface.

- Um Interface: The interface between the mobile device and the base station (radio interface).

- Abis Interface: The interface between the base station and the base station controller.

(c) With a diagram or figure, explain the process of an outgoing call.

- The mobile device sends a call request to the base station via the Um interface.

- The base station forwards the request to the Base Station Controller (BSC).

- The BSC communicates with the Mobile Switching Center (MSC).

- The MSC connects to the Public Switched Telephone Network (PSTN) to route the call.

- The call is established, and the voice/data transmission begins.

### Question 3

(a) Take three users and show the transmission and reception using spread spectrum:

- User 1: Data = 1 0, Spread Code = 0 1 0 1

- User 2: Data = 0 0, Spread Code = 0 0 0 1

- User 3: Data = 1 1, Spread Code = 1 1 1 1

Spread spectrum multiplies the data by the spread code for each user, creating a wider signal. At reception, the original data is recovered using the same spread code.

(b)
(i) List at least two technologies investigated in 5G.

- Massive MIMO (Multiple Input Multiple Output)

- Millimeter-Wave Communication

(ii) What is OFDMA?
OFDMA (Orthogonal Frequency Division Multiple Access) is a multi-user version of OFDM (Orthogonal Frequency Division Multiplexing). It allows multiple users to access the same frequency spectrum by assigning subsets of subcarriers to each user.

(iii) Mention any two advantages and disadvantages of OFDMA.

Advantages:

- Higher spectral efficiency.

- Reduced latency for users.

Disadvantages:

- High computational complexity.

- Requires precise synchronization between users.

Let me know if you need further clarifications!

Certainly, let's tackle the exam questions based on the provided image.

Question 1:

(a) Define roaming as used in mobile communication and give an example.

Answer:

In mobile communication, roaming refers to the capability of a mobile device to seamlessly connect and communicate via a network other than its home network. This typically happens when a user travels outside the geographical coverage area of their home network provider.

Example:

Imagine you're a resident of New York City and subscribe to a mobile carrier that primarily covers the city and its surrounding areas. If you travel to Los Angeles, your home network might not have coverage there. However, through roaming agreements between your carrier and other carriers in Los Angeles, your phone can automatically switch to one of those networks, allowing you to continue making calls, sending texts, and using data services as if you were still in New York.

(b) Differentiate between soft and hard-handover.

Answer:

Soft Handover: In soft handover, the mobile device is simultaneously connected to two or more base stations during a call. This ensures a smooth transition between cells as the device moves, minimizing call interruptions.

Hard Handover: In hard handover, the connection to the old base station is completely broken before a new connection to the new base station is established. This can lead to brief call interruptions during the handover process.

(c) Compare between GPRS and EDGE in terms of modulation type, symbol rate, modulation bit rate, and radio data rate.

Answer:

| Feature | GPRS | EDGE |
| --- | --- | --- |
| Modulation Type | GMSK | 8PSK |
| Symbol Rate | 270 kbps | 270 kbps |
| Modulation Bit Rate | 8 kbps | 12 kbps |
| Radio Data Rate | Up to 171.2 kbps | Up to 473.6 kbps |

Question 2:

(a) Abbreviate the following as in GSM network structure:

- NSS: Network Switching Subsystem

- ELR: Equipment Identity Register

- BSC: Base Station Controller

- GMSC: Gateway Mobile Switching Center

(b) Differentiate between Abis interface and G interface.

Answer:

- Abis Interface: This interface connects the Base Station Transceiver (BTS) to the Base Station Controller (BSC). It handles radio-level functions and data transfer between the BTS and BSC.

- G Interface: This interface connects the Base Station Controller (BSC) to the Mobile Switching Center (MSC). It handles call setup, routing, and other control functions.

(c) With a diagram or figure, explain the process of an outgoing call.

Answer:

(d) Draw a simplified GSM Network Architecture diagram.

Answer:

Question 3:

(a) Tabulate differences of WiFi, WiMAX, Bluetooth, and Zigbee in terms of speed and bandwidth.

Answer:

| Technology | Speed | Bandwidth |
| --- | --- | --- |
| WiFi | High | High |
| WiMAX | High | High |
| Bluetooth | Medium | Low |
| Zigbee | Low | Low |

(b) Mention any three differences between CSMA/CA and CSMA/CD.

Answer:

- Carrier Sense: CSMA/CA uses Carrier Sense Multiple Access with Collision Avoidance, while CSMA/CD uses Carrier Sense Multiple Access with Collision Detection.

- Collision Handling: CSMA/CA uses techniques like backoff algorithms to avoid collisions, while CSMA/CD relies on detecting and recovering from collisions.

- Applications: CSMA/CA is commonly used in wireless networks, while CSMA/CD is typically used in wired networks.

Question 4:

(a) What is vestigial sideband suppressed carrier (VSBSC) modulation?

Answer:

VSBSC is a type of amplitude modulation where one sideband is partially removed, and the carrier is suppressed. This reduces the bandwidth requirement compared to conventional amplitude modulation.

(b) Mention any three advantages and disadvantages of VSBSC.

Answer:

Advantages:

- Bandwidth Efficiency: VSBSC uses less bandwidth than conventional AM.

- Power Efficiency: VSBSC can be more power efficient than some other modulation techniques.

- Improved Signal-to-Noise Ratio: VSBSC can provide a better signal-to-noise ratio than AM in certain conditions.

Disadvantages:

- Complexity: VSBSC receivers can be more complex to implement than AM receivers.

- Distortion: VSBSC signals can be more susceptible to distortion during transmission.

- Limited Use Cases: VSBSC is primarily used in analog television broadcasting and some specialized applications.

Additional Notes:

- The provided image appears to be an exam paper or assignment.

- The quality of the image is somewhat low, making it difficult to read some parts clearly.

- The answers provided are based on the information available in the image and general knowledge of the topics.

Let me know if you have any further questions or require clarification on any specific aspect!