QUESTION

Using the public IP address of 192.168.2.1 and Network devices design the network of the Unit under DVC-ARC includes administration building and Dispensary for the Colleges.

Assume each college has four departments and each has 45 staff

Assume each non-college has two departments and each has 10 staff

The assigned task belongs to the non- college

“CENTRE FOR VIRTUAL AND CONTINUING EDUCATION” this is the non- college with two departments namely;

- Virtual education

- It offers two programs

- Online non degree

- Online degree

- Continuing education

- It offers two programs

- Face to face courses

- Online courses

In order to design the networks for the two departments to communicate each other and interact with the internet from the given public address 198.168.2.1 the following consideration should be taken

Subnetting: we need to do subnetting so as to divide the available ip ranges for both departments.

Routing: the aim of routing is to ensure each department communicate with one another.

Also network security devices such as firewall and VLANS should be used to separate each department traffic.

NETWORK REQUIREMENTS

- Public IP address 192.168.2.1

- Number of departments (2). Each department has 10 staffs, each staff require device like computer

NETWORK INFRASTRUCTURE

- Router: to connect to the internet

- Switch: to connect devices in each department

Addressing and subnetting

- Since the number of staffs for each department is 10 , that means for two departments total number of staffs will be 20,

- Apart from number of computer of each staffs, also each department may have other devices like printer.

Subnetting

Given IP address 192.168.2.1

Default subnet mask 255.255.255.0

If each department have 10 staffs, means number of host for each should be at least 10

128  64  32  16  /8  4  2  1 (host part)

New subnet mask 255.255.255.240/28

| Subnet | Network prefix | Usable IP address | Broadcast address |
| --- | --- | --- | --- |
| 1st subnet | 192.168.2.0 | 192.168.2.1 – 192.168.2.14 | 192.168.2.15 |
| 2ndsubnet | 192.168.2.16 | 192.168.2.17 – 192.168.2.30 | 192.168.2.31 |