ET 8402 MOBILE COMMUNICATION

TUTORIAL QUESTIONS

- Define small-scale fading. Clearly distinguish it from large-scale fading.

- Explain the physical mechanisms that cause multipath propagation in mobile radio channels.

- Describe the concept of a time-varying channel impulse response and explain why it is important in mobile communications.

- What is a power delay profile (PDP)? How is it obtained in practical channel measurements?

- Explain the significance of:

  - (a) Mean excess delay

  - (b) RMS delay spread

- Define coherence bandwidth and explain its relationship with RMS delay spread.

- Explain the difference between:

  - (a) Flat fading and frequency-selective fading

  - (b) Fast fading and slow fading

- What is Doppler shift? Explain how user mobility affects the received signal.

- Under what propagation conditions does Rayleigh fading occur?

- Explain the Ricean fading model and define the Ricean K-factor.

- Starting from a multipath channel model, derive the expression for the channel impulse response of a time-varying wireless channel.

- Show how coherence time is related to Doppler spread in a mobile radio channel.

- Derive the approximate relationship between coherence bandwidth and RMS delay spread.

- Explain mathematically why Rayleigh fading is a special case of Ricean fading.

- Using signal bandwidth and coherence bandwidth concepts, explain when inter-symbol interference (ISI) occurs.

- A mobile user is moving at a speed of 72 km/h while operating at a carrier frequency of 2 GHz.

(a) Calculate the maximum Doppler shift.

(b) Estimate the coherence time of the channel.

- A wireless channel has an RMS delay spread of 2 µs.

(a) Estimate the coherence bandwidth (50% correlation).

(b) Determine whether a signal of bandwidth 100 kHz will experience flat or frequency-selective fading.

- A digital communication system transmits symbols at a rate of 500 ksps over a multipath channel with a maximum excess delay of 4 µs.

(a) Determine whether ISI will occur.

(b) Suggest one suitable mitigation technique.

- A mobile radio channel is characterized by a Ricean K-factor of 8 dB.

(a) Explain what this value indicates about the propagation environment.

(b) State how system performance compares with a Rayleigh channel.

- A user operates in an urban environment where the coherence bandwidth is 150 kHz and the signal bandwidth is 1 MHz.

(a) Identify the type of fading experienced.

(b) Recommend a suitable transmission technique to mitigate the fading.

- Explain how OFDM reduces the effects of frequency-selective fading in broadband wireless systems.

- Describe the role of diversity techniques in combating small-scale fading. Give two practical examples.

- Explain how MIMO systems improve reliability and data rates in fading channels.

- Discuss the impact of small-scale fading on:

(a) Bit Error Rate (BER)

(b) Quality of Service (QoS)

- Consider a mobile communication system in Tanzania operating in a dense urban area. Discuss suitable fading mitigation techniques for reliable service delivery.

- With the aid of neat sketches, explain:

(a) Power delay profile

(b) Doppler power spectrum

- Compare Rayleigh and Ricean fading channels in terms of:

LOS condition

Statistical distribution

Practical applications

- Explain why equalization is essential in wideband mobile communication systems.