# Assignment 3: WAN Technologies – Scenario Answers

Scenario 1: Virtual Circuits & DLCIs (Frame Relay)

1. Office A and B use different DLCI numbers because DLCIs are locally significant; the Frame Relay network maps them internally.

2. The virtual circuit used is a Permanent Virtual Circuit (PVC).

3. Communication between A and B will not be affected because each PVC operates independently.

Scenario 2: CIR, Congestion & DE Bit

1. Frames sent above CIR are marked DE and may be dropped during congestion.

2. The DE bit lowers priority, making frames more likely to be discarded.

3. Off-peak hours have less traffic, so congestion is minimal.

Scenario 3: FECN & BECN Congestion Control

1. BECN indicates congestion in the reverse direction.

2. FECN indicates congestion in the forward direction.

3. Routers should reduce transmission rate.

Scenario 4: ATM Cells and Fixed Cell Size

1. ATM uses fixed 53-byte cells to ensure predictable delay.

2. Fixed cells reduce jitter for voice and video.

3. Larger cells increase delay and jitter.

Scenario 5: QoS and ATM Traffic Classes

1. VBR is best for video lectures.

2. VoIP requires lower delay than web traffic.

3. ATM ensures QoS via traffic shaping and policing.

Scenario 6: Virtual Paths and Virtual Channels

1. VPs group VCs for efficient management.

2. Bundling reduces switching complexity.

3. All VCs fail if the VP fails.

Scenario 7: SONET Fixed Frame Structure

1. SONET uses fixed frames for predictable timing.

2. Synchronization supports real-time traffic.

3. Sync problems cause delay and jitter.

Scenario 8: Bandwidth Allocation in SONET

1. SONET uses TDM to allocate bandwidth.

2. It guarantees fixed bandwidth.

3. Excess traffic is delayed or dropped.

Scenario 9: Protection Switching

1. Traffic is rerouted to backup paths.

2. Ring topology provides redundancy.

3. Fast recovery prevents service disruption.

Scenario 10: MPLS Label Switching

1. Labels reduce processing overhead.

2. Label switching speeds forwarding.

3. Latency increases without labels.

Scenario 11: MPLS Traffic Engineering

1. MPLS TE reserves bandwidth.

2. MPLS enforces QoS end-to-end.

3. Misconfigured QoS causes poor performance.

Scenario 12: MPLS VPNs

1. VRFs allow overlapping IPs.

2. Labels ensure traffic separation.

3. Redundancy limits failure impact.

Scenario 13: Cost vs Performance WAN Design

1. MPLS, SD-WAN, and broadband VPN reduce costs.

2. Lower-cost WANs may reduce reliability.

3. QoS prioritizes video traffic.

Scenario 15: QoS and Application Prioritization

1. MPLS and SD-WAN suit real-time traffic.

2. Voice/video get highest priority.

3. Misconfigured QoS risks patient care.

Scenario 16: Reliability and Redundancy

1. Use redundant links and ISPs.

2. Implement failover routing.

3. Higher reliability increases cost.